GESTLINK

| ARCHITECTURAL DESIGN DOCUMENT |  |  |
| --- | --- | --- |
|  | Version 1.0 | Page 3 of 3 |

Architectural Design Diagram

| Application | Gesture Classification Logic (A) | Local Feedback Mapping (B) | MQQT Data Staging (C) | System Monitoring (D) |
| --- | --- | --- | --- | --- |
| Drivers | Sensor (E) | GPIO (F) | TCP/IP Stack (G) | OS Layer (H) |
| HAL | I2C / SPI (I) | GPIO (LEDs) (J) | TIM (K) | Ethernet MAC (L) |
| STM 32 (M) |  |  |  |  |

Interactions

M --> I

M --> J

M --> K

M --> L

I --> E

J --> F

E --> A

F --> B

K --> H

L --> G

G --> C

H --> A

H --> B

H --> C

H --> D

A --> B

A --> C

B --> J

C --> G

D --> G

D --> F

D --> E

Layer 2 : Hardware Abstraction Layer (HAL)

| Component | Role | Upstream Interaction |
| --- | --- | --- |
| I (I2C / SPI) | Handles low-level serial communication protocols necessary to transfer raw data between the STM32 and the PAJ7660 Sensor. | Feeds the Sensor Driver (E). |
| J (GPIO - LEDs) | Provides direct control over the physical pins used to drive the High-Luminosity LEDs. | Controls the GPIO Driver (F), which handles local feedback. |
| K (TIM) | Manages system timers, interrupts, and scheduling ticks. | Crucial for the reliable operation of the OS Layer (H). |
| L (Ethernet MAC) | Manages the physical link and data framing required for all network traffic (Wired Ethernet). | Feeds the TCP/IP Stack (G). |

Layer 3 : Middleware / Drivers

| Component | Role | Upstream/Downstream |
| --- | --- | --- |
| E (Sensor Driver) | Manages the specific initialization, configuration, and data retrieval for the PAJ7660 sensor. It retrieves the pre-classified Gesture ID from the sensor. | Interfaces with HAL I2C/SPI (I) and supplies data to Classification Logic (A). |
| F (GPIO Driver) | Provides simple, logical functions (e.g., set_leds_by_count(N)) to control the LEDs. It maps the abstract request from the application to the physical pin toggles via the HAL. | Interfaces with HAL GPIO (J) and takes commands from Feedback Mapping (B) and System Monitoring (D). |
| G (TCP/IP Stack - LWIP) | The Lightweight Internet Protocol stack. It handles the necessary TCP and IP layers to enable reliable, routable communication over Ethernet. | Interfaces with HAL Ethernet MAC (L) and supports the MQTT Staging (C) block. |
| H (OS Layer - FreeRTOS) | The Real-Time Operating System that manages concurrency. It ensures tasks (A, B, C, D) run reliably with necessary priority, crucial for the <=200 ms latency requirement. | Built on HAL TIM (K) and provides task scheduling services to all blocks in the Application Layer (A, B, C, D). |

Layer 4 : Application Layer

| Component | Role | Inputs / Outputs |
| --- | --- | --- |
| A (Gesture Classification Logic) | The main control task. It retrieves the pre-classified Gesture ID from the Sensor Driver (E), validates it, and triggers the necessary outputs. | Input: Sensor Driver (E). Output: Feedback Mapping (B) and MQTT Staging (C). |
| B (Local Feedback Mapping) | Implements the low-latency visual confirmation. It receives the classified ID and calls the GPIO Driver (F) to display the corresponding LED pattern immediately. | Input: Classification Logic (A). Output: GPIO Driver (F) (via J). |
| C (MQTT Data Staging) | Handles the protocol translation and network communication. It formats the Gesture ID into a reliable QoS 1 MQTT payload and initiates transmission. | Input: Classification Logic (A). Output: TCP/IP Stack (G). |
| D (System Monitoring) | Manages high-level system checks, including network status, sensor health, and watchdog timer maintenance. It triggers fault indicators upon failure. | Monitors TCP/IP Stack (G) and Sensor Driver (E). If failure, triggers the alert via the GPIO Driver (F). |